MBSD Assignment #3 A.Y. 2024/25

# Purposes

- Perform some parts of the Functional and Technical Safety Concept analysis, according to ISO26262, of a “one pedal controller” for a car.

- Implement some of the safety concepts in the Simulink model of the controller developed in Laboratory #2.

- Perform unit and integration tests on the implemented safety-related functionalities.

It is available an example of a Functional Safety Concept for the item Front Light Manager (FLM).

The deliverable, composed of

- the report (the following pages of this document)

- the Simulink models on where the safety concepts have been implemented

- all the needed files to replicate the software testing results

has to be provided as a .ZIP file (do not use other compression formats) up to May 14th at 23:59. It shall also contain a brief report explaining the design of the controller using the following template.

It is sufficient that only one of the group members uploads it.

IMPORTANT:

For making the Functional Safety Concept coherent between the groups of the course, consider as ASIL C all the safety goals related to unintended acceleration (those leading to an increase of the vehicle’s speed modulus) and as ASIL B the warnings to the driver and the unintended deceleration (those leading to a decrease of the vehicle’s speed modulus).

# Model-Based Software Design, A.Y. 2022/23

# Laboratory 3 Report

## Components of the working group (max 2 people)

- Federico Bena, 328197

- Michele Lauriola, 332679

Functional Safety Concept

One pedal

# Functional safety architecture

Figure 1 Functional safety architecture

# Attributes of the Safety Goals

Fill the table with the safety goal descriptions

| SG 1 | The item shall compute correctly the torque requested by the driver. |
| --- | --- |
| SG 2 | The item shall monitor that the transmission mode displayed is the correct one. |
| SG 3 | The item shall transit to safe state when a misbehavior is detected. |

Fill in the attribute/parameters of the safety goal

| Safety goal | Attributes/Parameters of the safety goal |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Integrity (ASIL) | Safe state | Fault tolerance time | Warning concept | Degradation concept |
| SG 1 | C | Prevent unintended or insufficient acceleration and deceleration. | 100 ms | Alert the driver of the malfunction by displaying a notification on the dashboard and emitting a beeping sound or an alarm. | No. |
| SG 2 | B | Prevent the wrong Transmission mode from being displayed. | 300 ms | Alert the driver of the malfunction by displaying a notification on the dashboard and emitting a beeping sound or an alarm. | No. |

# Functional (and technical) safety requirements and allocation

|  |  | Define functional safety requirements |  | Allocation of requirements on systems and elements |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Safety requirements | Remark | If applicable, allocate the safety requirements to other Items / Systems | If applicable, allocate the safety requirements to equipment other technologies to minimize risk. That could be e.g. hydraulic, mechanical equipment |
| Safety goals | SG1: The item shall compute correctly the torque requested by the driver. | SR1: If one throttle pedal sensor is faulty, the system will compute the average of the other two working sensors. | The three sensors are independent from each other. | A warning message is displayed on the dashboard. | No. |
|  |  | SR2: If all the throttle pedal sensors are faulty (at least two of them), the transmission state changes to neutral. | The three sensors are independent from each other, moreover the torque request is set to 0. | The System must transit to safe state, and a notification about the fault is displayed on the dashboard accompanied by an alarm. | No. |
|  |  | SR3: The warning remains on until a correct measurement is detected. | No. | No. | No. |
|  |  | SR4: The system shall compare the torque requests against the expected values, in case of fault, the transmission state changes to neutral. | In case of Fault the torque request is set to 0. | The System must transit to safe state, and a notification about the fault is displayed on the dashboard accompanied by an alarm. | No. |
|  | SG2: The item shall monitor that the transmission mode displayed is the correct one. | SR5: If the displayed mode is different from the actual one the system must transit to safe state. | In the case of Fault, the torque request is set to 0 within a certain time tolerance. | The System must transit to safe state, and a notification about the fault is displayed on the dashboard accompanied by an alarm. | No. |
|  |  | SR6: The warning remains on until a correct mode is displayed. | No. | No. | No. |

# ASIL preliminary architecture

The purpose of this figure is to assign ASILs to the components indicated in Figure 1 on the functional safety architecture.

Figure 2 Preliminary architecture (without ASIL decomposition).

# Implementations

Figure 3: Implementation of Functional Redundancy and Plausability

Figure 3 shows that the Saturation blocks, used in the previous assignment, have been included as checks in the Functional Redundancy and Plausability block.

Figure 4: Implementation at Harness level

In the following table, the new variables, used for the Functional Redundancy and Plausability block, are defined:

| Name | Type | Data Type | Dimension | Min | Max |
| --- | --- | --- | --- | --- | --- |
| Limiter | Input | Boolean | 1x1 | 0 | 1 |
| Fault | Input | Boolean | 1x1 | 0 | 1 |
| PedalError | Output | Single | 1x1 | 1 | 4 |
| Warning | Output | Enum: WarningType* | 1x1 | Off(0) | Red(2) |

*classdef WarningType < Simulink.IntEnumType

enumeration

Off(0)

Yellow(1)

Red(2)

end

end

## Functional redundancies

Indicate which components of the functional safety architecture are implemented with redundancies.

Functional redundancies are implemented, as safety measures, through two additional sensors that measure independently the position of the throttle pedal allowing to detect potential failures or outliers.

Within the Functional Redundancy and Plausibility block, the Matlab function Voter_TPP receives the input measurements from the three sensors (ThrottlePedalPosition1, ThrottlePedalPosition2 and ThrottlePedalPosition3) and it performs the following actions:

- Initializes the variables Warning and Fault used respectively to alert the driver of the presence of an anomaly or a more severe fault through the dashboard and to transit into a safe state.

- Computes, in absolute value, the differences between each pair of measurements (Diff_1_2, Diff_1_3 and Diff_2_3).

- Performs the various if-cycles to check the consistency of the measurements by comparing the computed differences to a fixed threshold of 2 %, returning three possible outcomes:

  - The three measurements are consistent. The TPP (ThrottlePedalPosition) output is computed as the average of the three measurements (no Warning).

  - One of the three measurements deviates from the other two. The TPP output is given as the average of the latter, the PedalError shows the number of the faulty sensor and the variable Warning is set to ‘1’, corresponding to the yellow Lamp on the dashboard which means that the vehicle is not working properly but still able to provide its functions: the driver must service the vehicle as soon as possible.

  - The three measurements are inconsistent with each other. The PedalError shows the number ‘4’, the Fault variable is set to “1” so the system transits into a safe state and the Warning is set to ‘2’, which means that all the sensors are inconsistent between each other and so the driver must stop immediately the vehicle and call assistance.

## Implemented plausibility checks

Indicate which signals are subjected to plausibility checks. Indicate also the checks needed for the channels with redundancies in place.

Plausibility checks are implemented to verify the consistency of the input signals coming from CAN. Such evaluations are performed through MATLAB function blocks included in the previously introduced Functional Redundancy and Plausibility block:

- BPP_Plausability: if the data BrakePedalPressed is not a Boolean value, Fault_BPP is set to “1” and Warning_BPP is set to ‘2’.

- ATSS_Plausability: if the data AutomaticTransmissionSelectorState is not an integer between 0 and 4, corresponding to the available Transmission modes, the Fault_ATSS variable is set to “1” and Warning_ATSS is set to ‘2’.

- TR_Plausability: the consistency of the controller outputs TorqueRequest_Nm and AutomaticTransmissionState is checked. When the torque is: zero OR a positive value in ‘D’ or ‘B’ mode OR a negative value in ‘R’ or ‘B’ mode, if the torque exceeds the range [– 80,80] Nm, Limiter_TR is set to “1” and Warning_TR is set to ‘1’, otherwise Fault_TR is set to “1” and Warning_TR is set to ‘2’.

- VS_Plausability: if the data coming from the VehicleSpeed_km_h excedees the range [-60,240] km/h the variable Limiter_VS is set to “1” and Warning_VS is set to ‘1’.

Figure 5: Functional Redundancy and Plausability Block

As can be noticed in Figure 5, it should be pointed out that:

- For the several types of faults and limiters, logical operators OR are implemented returning the two generic Boolean variables Fault and Limiter.

- Since the warnings are Enum: WarningType, a Matlab function, that checks them all and returns as output the higher value of Warning, is implemented.

Figure 6 shows the addition of the Fault and Limiter variables in the transitions (highlighted in yellow) within the Controller State Machine:

- “Fault == 1” is added as a logical OR to all the transitions towards the Neutral State, including from ‘Brake’ to ‘Drive’.

- “Fault == 0” is added as a logical AND to all the transitions from ‘Neutral’ to the other States, including from ‘Drive’ to ‘Brake’.

This implementation makes sure that the current State is set to ‘Neutral’ if the Fault is active, and that the item is allowed to change State otherwise.

As concerns the Limiter new sub-states have been created to prevent the controller from leaving its current state while still maintaining the limiter function for speed and torque:

- “Limiter == 1” is added as a logical OR to all the transitions towards the “Limiter” Sub-State, or to the “EmergencyStop” for the ‘Brake’.

- “Limiter == 0” is added as a logical AND to all the transitions from “Limiter” or “EmergencyStop” to the other Sub-States.

Figure 6: Fault and Limiter variables in the State Machine's Transitions

# Software testing

## Implemented unit tests

Describe in English the test performed to verify the correct functionality of the safety mechanism implemented.

Simulink Coverage is used for all the subsystem blocks, shown on Figure 5, to check whether the implemented Matlab functions result in 100% coverage.

Following are: the tables concerning the values used for the input blocks Repeating Sequence Stair, the sample times and the simulation time (adapted according to each unit test) and the Figures to presents the coverage results and the trends of the variables of interest, obtained through a Scope block.

- Voter_TPP Unit Test

| Simulation Time [s] |  |  | 6 |
| --- | --- | --- | --- |
| Repeating Sequence Stair | Vector of values | Sample Time [s] |  |
| ThrottlePedalPosition1 | [0.8].' | 6 |  |
| ThrottlePedalPosition2 | [0.8 0.81 0.81 0.9 0.62 0.9 ].' | 1 |  |
| ThrottlePedalPosition3 | [0.8 0.79 0.7 0.79 0.6 0.6].' | 1 |  |

Figure 7: Coverage of Voter_TPP

Figure 8: Input (Top) & Output (Bottom) Scope Voter_TPP

Figure 7 and Figure 8 show that block Voter_TPP behaves as expected.

- BPP_Plausabiliy Unit Test

| Simulation Time [s] |  |  | 5 |
| --- | --- | --- | --- |
| Repeating Sequence Stair | Vector of values | Sample Time [s] |  |
| BrakePedalPressed | [0 2 1 -0.5 0].' | 1 |  |

Figure 9: Coverage of BPP_Plausability

Figure 10: Input & Output Scope BPP_Plausability

Figure 9 and Figure 10 show that block BPP_Plausability behaves as expected.

- ATSS_Plausabiliy Unit Test

| Simulation Time [s] |  |  | 11 |
| --- | --- | --- | --- |
| Repeating Sequence Stair | Vector of values | Sample Time [s] |  |
| AutomaticTransmissionSelectorState | [-1 0 0.5 1 1.5 2 2.5 3 3.5 4 5].' | 1 |  |

Figure 11: Coverage of ATSS_Plausability

Figure 12: Input & Output Scope ATSS_Plausability

Figure 11 and Figure 12 show that block ATSS_Plausability behaves as expected.

- VS_Plausabiliy Unit Test

| Simulation Time [s] |  |  | 16 |
| --- | --- | --- | --- |
| Repeating Sequence Stair | Vector of values | Sample Time [s] |  |
| Velocity | [-80 -80 -60 -60 -30 -30 0 0 80 80 200 200 240 240 280 280 ].' | 1 |  |

Figure 13: Coverage of VS_Plausability

Figure 14: Input & Output Scope VS_Plausability

Figure 13 and Figure 14 show that block VS_Plausability behaves as expected.

- TR_Plausabiliy Unit Test

| Simulation Time [s] |  |  | 25 |
| --- | --- | --- | --- |
| Repeating Sequence Stair | Vector of values | Sample Time [s] |  |
| TorqueRequest_Nm | [-90 -45 0 45 90].' | 1 |  |
| AutomaticTransmissionState | [0 1 2 3 4].' | 5 |  |

Figure 15: Coverage of TR_Plausability

Figure 16: Input & Output Scope TR_Plausabilitys

Figure 15 and Figure 16 show that block TR_Plausability behaves as expected.

- Warning_Detection Unit Test

| Simulation Time [s] |  |  | 30 |
| --- | --- | --- | --- |
| Repeating Sequence Stair | Vector of values | Sample Time [s] |  |
| Warning_TPP | [0 1 2].' | 2 |  |
| Warning_BPP | [0 0 2].' | 2 |  |
| Warning_ATSS | [0 0 2].' | 4 |  |
| Warning_VS | [0 1 0 0 1].' | 1 |  |
| Warning_TR | [0 1 0 2].' | 2 |  |

Figure 17: Coverage of Warning_Detenction

Figure 18: Input & Output Scope Warning_Detenction

Figure 17 and Figure 18 show that block Warning_Detenction behaves as expected.

## Implemented integration tests

Describe, in English, the scenarios tested at the integration level to verify the proper integration between the various units implementing the safety mechanisms.

Figure 19: Harness with step used to corrupt some signal for Testing purposes

Figure 19 shows how the harness environment has been modified, with the addition of some step block, in order to corrupt and then readjust some input signal, so simulating the behavior of the controller in the presence of disturbed signal.

- Scenario 1: Corruption at the throttle pedal signals level.

Figure 20: Result whit only the Throttle Pedal sensors corruption

As shown in Figure 20, the third sensor is corrupted at 10 s and the vehicle displayes a yellow warning remaining in the same AutomaticTransmissionState. When the first sensor is corrupted at 20 s, the vehicle displays a red warning entering in the Safe State (AutomaticTransmissionState = Neutral).

- Scenario 2: Corruption at the brake pedal signal level.

Figure 21: Result whit only the BrakePedalPressed signal corrupted

As shown in Figure 21, at 10 s the BreakePedalPressed signal is corrupted, the vehicle displayes a red warning entering in the Safe State (AutomaticTransmissionState = Neutral).

- Scenario 3: Corruption at the speed signal level.

Figure 22: Result whit only the Feedback Speed signal corrupted

As shown in Figure 22, at 10 s the feedbacked VehicleSpeed_km_h signal is corrupted, the vehicle displayes a yellow warning remaining in the same AutomaticTransmissionState.

- Scenario 4: Corruption of all the signals.

Figure 23: Result whit all the signals corrupted

Figure 23 shows what happens in the case in which BrakePedalPressed signal is corrupted at 55 s (achieving  the same result of Figure 21), while the ThrottlePedalPosition and the VehicleSpeed_km_h corruption are set in a way to overlap them and to show the right functioning of the Detection_Warning block.

- Scenario 5: Original test (no signal corruption).

Figure 24: Sparklines results

Figure 25: Errors and Warnings

Figure 24 and Figure 25 show the behaviour of the system in the original test conditions resulting clearly that, although a whole part dedicated to checking the validity of input signals has been added to the controller, the dynamic response of the system remains unchanged when compared to the previous report.

From that it is possible to state that the changes made do not affect the response any way.

See document 02-iso26262.pdf, slides 89, 90, 91, 92, 93.

In the ISO26262 the implementations are based on a document called Technical Safety Concept, but for simplicity we move straight from the Functional Safety Concept to software implementations.

A guideline for the implementation phase can be found in the document 02-iso26262.pdf from slide 81, in particular slide 86.